Experiment #3

“Verification of Castigliano’s Theorem

And

Maxwell Law of Reciprocal Deflection by bending of beams”

Objective:-

The objective of experiment is to investigate the castigliano’s method of calculating beam displacement to observe the reciprocity of displacement as stated by Maxwell.

Apparatus:-

- Steel beam

- Deflection of beam apparatus

- End support with clamping fixture and hanger

- End support with roller fixture and hanger

- Weights

Related Theory

Strain Energy For Axially Loaded Bar:-

F



A

L

σaxial=FA;Δ=FLAE;U=12FΔ=F2L2AE

F= Axial Force (Newtons, N)

A = Cross-Sectional Area Perpendicular to “F” (mm2)

E = Young’s Modulus of Material, MPa

L = Original Length of Bar, mm

Castigliano’s Second Theorem:-

- The term “force” may be used in its most fundamental sense and can refer for example to a Moment, M, producing a rotation, q, in the body.

- Castigliano’s 2nd theorem can be used to determine the deflections in structures (e.g. trusses, beams, frames, shells) and we are not limited to applications in which only 1 external force or moment acts.

- Furthermore, we can determine the deflection or rotation at any point, even where no force or moment is applied externally.

- Castigliano's method for calculating displacements is an application of his second theorem, which states:

“If the strain energy of a linearly elastic structure can be expressed

as a function of generalized force Qi; then the partial derivative

of the strain energy with respect to generalized force gives the

generalized displacement qi in the direction of Qi.”

θn=∂U∂Mn

Castigliano's method for calculating forces is an application of his first theorem, which states:

“If the strain energy of an elastic structure can be expressed as a

function of generalized displacement qi; then the partial derivative

of the strain energy with respect to generalized displacement gives

the generalized force Qi.”

Maxwell Law of Reciprocal Deflection:-

“The deflection of a point “D” on a structure or beam due to a unit load acting at “D” is equal to the deflection at point “D” due to unit load acting at point “C”.”

δcd  = δdc

“The rotation of a point “B” on a structure or beam due to a unit couple acting at “A” is equal to the rotation at point “A” due to unit couple acting at point “A”.”

θab = θba

“If a unit force and a unit couple are acting at separate points on structure or beam than the rotation in radians at point “A” on a structure due to unit load acting on a beam at point “C” is equal to the deflection at point “C” when a unit couple is acting on a point “A”.”

θad = δda

Arrangement of Apparatus

Procedure:-

- Adjust the apparatus in the frame

- Adjust the beam in the frame

- Attach the load hangers on the beam and make their load zero by setting dial gage reading to a known value

- Change the end conditions according to requirement that roller, hing or fix.

- Apply the loads on the hangers

- Note the deflection and rotation of the beam

- Note down the final reading in the observation and calculation table

Calculations of Deflections & Rotations

Fixed ended moments = 10 x 0.48 x 0.482 / 0.962

= 1.2 Nm

Portion           Origin             Limit                         M                               m

AC                   A                0-0.48                     5 x – 1.2                0.5 x – 0.125

BC                   B                0-0.48                     5 x – 1.2                0.5 x – 0.125

Δ = 1 / EI [0ƒ0.48 (5x – 1.2)(0.5x – 0.125) dx  +  0ƒ0.48 (5x – 1.2)(0.5x – 0.125) dx ]

Δ = 0.04608 / EI

E = 200 GPa

I = 5.625 x 10-11 m4

Δ = 4.096 mm

Fixed ended moments = 20 x 0.48 x 0.482 / 0.962

= 12.4 Nm

Portion           Origin             Limit                         M                               m

AC                   A                0-0.48                     10 x – 2.4                0.5 x – 0.125

BC                    B                0-0.48                     10 x – 2.4               0.5 x – 0.125

Δ = 1 / EI [0ƒ0.48 (10x – 2.4)(0.5x – 0.125) dx  +  0ƒ0.48 (10x – 2.4)(0.5x – 0.125) dx ]

Δ = 0.09216 / EI

E = 200 GPa

I = 5.625 x 10-11 m4

Δ = 8.192 mm

δ

Compatibility Equation

Δ = R x δ

Unit load analysis:-

δbc =  0.48 x 0.48 x 0.24 + 0.48 x 0.48 x 0.32 / 2

= 0.09216

δbb = 0.96 x 0.96 x 0.64 / 2

= 0.294912

From compatibility equation

R = 3.125 N

By static Equations

Ra = 6.875 N

Ma = -1.8 N-m

By solving the same beam by unit load

Rb’ = 0.3125 N

Ra’ = 0.6875 N

Ma’ = -0.18 N-m

Portion         Origin         Limit                    M                                    m

AB                  A             0-0.48         -1.8 + 6.875 x              -0.18 + 0.6875 x

BC                   B            0-0.48               3.125 x                            0.3125 x

By solving the above data by castiglano’s theorm

Δ = 0.08064 / EI

E = 200 x 106 MPa

I = 5.625 x 10-11 m4

Δ = 7.168 mm

Portion         Origin         Limit                    M                             m

AB                  A            0-0.48          -1.8 + 6.875 x            0.5-1.5625 x

BC                  B            0-0.48                 3.125 x                    1.5625 x

By solving the above data by castiglano’s theorm

θ =            / EI

E = 200 GPa

I = 5.625 x 10-11 m4

θ =            rad

Observation & Calculation Table

| Loading | LHE |  | Mid Span |  |  | RHE |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DGR | Rotation ( rad) | DGR | Deflection (mm) |  | DGR | Rotation ( rad) |  |
|  |  |  |  | Exp. | Analy. |  | Exp. | Analy. |
| 0 | Locked |  | 800 | 7.34 | 7.168 | 800 | 0.0248 |  |
| 10 (N) |  |  | 66 |  |  | 552 |  |  |
| 0 | Locked |  | 1680 | 4.43 | 4.096 | Locked |  |  |
| 10(N) |  |  | 1237 |  |  |  |  |  |
| 20(N) |  |  | 795 | 8.85 | 8.192 |  |  |  |

Maxwell Law of Reciprocal Deflection

Apparatus Arrangement

Using conjugate beam method

θac = 0.576 / EI

= 0.0512 rad

θbc = 0.576 / EI

= 0.0512 rad

δdc = 0.576 x 0.24 – 1.2 x 0.242 /6

= 0.127 / EI

= 11.29 mm

Using conjugate beam method

θad = 0.36 / EI

= 0.032 rad

θbd = 0.504 / EI

= 0.0448 rad

δcd = 0.36 x 0.48 – 1.2 x 0.482 /6

= 0.127 / EI

= 11.289 mm

Using conjugate beam method

θab = 1600 / EI

= 0.000142 rad

δcb = 1.6 x 0.48 – 5 x 0.482 /6

= 0.576 / EI

= 0.0512 mm

δdb   = 1.6 x 0.72 – 7.5 x 0.722 /6

= 0.504 / EI

= 0.0448 mm

Using conjugate beam method

θba = 16 / EI

= 0.000142 rad

δca = 1.6 x 0.48 – 5 x 0.482 /6

= 0.576

= 0.0512 mm

δda = 1.6 x 0.24 – 2.5 x 0.242 /6

= 0.36 / EI

= 0 .032 mm

Observation & Calculation

| Point | Loading | LHE |  | Mid Span |  | 3/4 Span |  | RHE |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | DGR | Rotation ( rad) | DGR | Deflection (mm) | DGR | Rotation ( rad) | DGR | Rotation ( rad) |
| C | 0 | 1183 | 0.049 | 1670 | 15.8 | 1534 | 10.94 | 1105 | 0.037 |
|  | 10(N) | 1678 |  | 92 |  | 440 |  | 635 |  |
| D | 0 | 1201 | 0.028 | 1610 | 10.3 | 1489 | 8.83 | 1186 | 0.051 |
|  | 10(N) | 1488 |  | 573 |  | 606 |  | 674 |  |
| B | 0 | 1198 | 0.011 | 1502 | 4.8 | 1498 | 4.96 | 1090 | 0.0355 |
|  | 10(N-m) | 1079 | 0.000073 | 1982 | 0.032 | 1994 | 0.033 | 1445 | 0.0002370 |
| A | 0 | 1078 | 0.0283 | 1296 | 6.39 | 1563 | 3.85 | 1121 | 0.0157 |
|  | 10(N-m) | 788 | 0.000189 | 1935 | 0.043 | 1948 | 0.026 | 1278 | 0.000105 |

Results:-

- δcd  = δdc

- δac  = δca

- θab = θba

- θad = δda

- θbc = δcb

- θbd = δdb

Precautions:-

The following precautions should be taken in order to perform the experiment properly in the lab

- The beam should be properly adjusted in the frame

- The hangers should be located at the proper place

- The distance b/w the supports should be accurate

- During the performance the movement of apparatus should be avoided.

- The load should be applied slowly.

- Reading should be taken carefully

Comments:-

There is a little difference b/w the experimental and the analytical values which may be due to the following reasons

- There is the friction b/w the parts of the apparatus

- The length of the beam may not be accurate and the loads may not be hanged at the accurate position supposed

- This error may be due to reason than the supports provided may not be properly set